MINI DISPLAY

INSTRUCTIONS

SOFTWARE INSTALLATION

1. Insert the floppy disk into drive ‘A` or ‘B`.

2. Left click once on the START button and then left click RUN.

3. In the command line OPEN type  a:install and press ENTER

4. When the install has finished left click once on the X at the top right corner to close off the install window.

5. Left click once on START point to SETTINGS and then point to TASK BAR OPTIONS and left click once.

5. Left click once on the TAB   START MENU PROGRAMS and then left click ADD.

6. In the command line type   C:\MINI\TERM90 and left click NEXT.

7. The next window will ask you where you would like to place this icon, from the tree displayed left click once on   the word START MENU then left click NEXT and again left click FINISH.

Congratulations you have just installed the terminal program and made an icon to start using you MINI DISPLAY.

# SETTING UP THE SIGN

- Plug the data cable into the serial port  COM2 on your PC.

- Plug the stereo jack into the back of the MINI DISPLAY.

- Plug the power connector into the back of the MINI DISPLAY and the power pack into the power point. DO NOT turn on the power at this stage.

- Left click START and point to TERM90 and then left click this icon to start the terminal program.

- When TERM90 starts you will see a blank screen with some menus DO NOT change any of these menus.

- Now turn on the power to your MINI DISPLAY and you will see a MENU displayed. You are now ready to start programming the MINI DISPLAY.

The Menu displayed is as follows.

To setup your message on the Mini Display Please use the following commands.

- Type new message

- Delete message

- Program message

- Message scroll speed = FAST

- Message scroll speed = MEDIUM

- Message scroll speed = SLOW

## Programming the MINI DISPLAY

- The CAPS LOCK key must be pressed to enable your text to be sent to the MINI DISPLAY.

- Press 1 and a sub menu will be displayed to enable you to enhance your message in BOLD or REVERSE text

Note: Model MD83 only supports NORMAL, BOLD and REVERSE text.

- Adjust the scroll speed by selecting option 4, 5 or 6 from the main menu Type in your text and you will see this displayed in real time on the MINI DISPLAY.

- Press CTRL+B before the text you want in BOLD, this will display a smiley face symbol or CTRL+R before the text you want in REVERSE, this will display a up & down arrow symbol and then press CTRL+E  to end the enhancements.

- DO NOT use the arrow keys to cursor UP, DOWN, LEFT or RIGHT this will corrupt the displayed text and you will have to start again. Use the backspace key to move over the text you want to change and retype your text.

- After your message is complete press ESC, you will see a prompt MESSAGE ACCEPTED. Now press  3 to program the MINI DISPLAY. You will see a prompt  MESSAGE PROGRAMMED. Press ESC to clear the screen and redisplay the main menu.

- If you want to change your message simply follow the steps under SETTING UP THE SIGN and then press 2 to delete your message and then follow the steps under Programming the MINI DISPLAY.

- Finally select FILE, EXIT and OK to close the terminal program, unplug your data cable and place the MINI DISPLAY in the appropriate location.