"Switchers Made Simple" V3.3

Supplemental Information (README.DOC file)

Table of Contents

1.0  Introduction

2.0  Outline of Program

2.1  Specifying the Converter

2.2  Calculation of Component Limit Values

2.3  Component Selection

2.4  Editing Component Values

2.5  Frequency Response Calculation

2.6  Thermal Calculations

2.7  Saving, Viewing, and Printing the File

3.0  Warning Messages

3.1  "Current or Voltage Limit Exceeded."

3.2  "Burst Mode Operation."

3.3  "Short Circuit Runaway."

3.4  "Possible Subharmonic Oscillation."

3.5  "Maximum Duty-cycle Exceeded."

4.0  General Comments

4.1  Standard Units

4.2  IC Options

4.3  Output Accuracy

4.4  Output Capacitor ESR

4.4.1  Effect of Temperature on Capacitor ESR

4.4.2  ESR and Output Ripple Voltage

4.4.3  Editing Output Capacitor ESR

4.5  Standard/Custom Inductors and Transformers

4.6  Prime Output and Turns Ratio in Multiple Output Flyback

Converters.

4.7  Crossover Frequency and Phase Margin

4.8  Continuous/Discontinuous Operation

4.9  Low Power Converters and Discontinuous Operation

4.10 Components Not Found in the Database

5.0  A Word of Caution

6.0  Comments and Suggestions

7.0  Suggested Reading

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1.0  INTRODUCTION

The "Switchers Made Simple" program is an expert system that automates

the design of switching regulators based on SIMPLE SWITCHER (TM) power

converters.  The following topologies can be designed with this

program: buck and buckboost (invert) regulators with the LM2574/75/76

family, and boost and multiple-output flyback regulators with the

LM2577 family.  The program includes all available packages for SIMPLE

SWITCHER power converter product line:

K = TO-3 for LM1575 and LM1577 (military temperature range

versions of LM2575 and LM2577)

T = TO-220

N = 8-pin DIP for LM2574, 16 pin power DIP for LM2575 and LM2577

M = 14-pin SO for LM2574, 24 pin power SO for LM2575 and LM2577

2.0  OUTLINE OF PROGRAM

2.1  Specifying the Converter

The first module of the program lets you input the specifications

of the converter you wish to design.  It is done via a menu, with

editing capability.  The feasibility of the specifications are

checked and a provision is made for editing them, should the

program alert the user to an unfeasible parameter set.

2.2  Calculation of Component Limit Values

The second module calculates the limit values of the regulator's

external components based on the specification and loop stability

requirements.  The stability analysis calculation is based on the

state-space-averaged model introduced by Dr. Middlebrook.

2.3  Component Selection

Next, the program chooses the actual components, according to the

calculated limits, from a built in database of standard components.

There are three manufacturers providing standard inductors and

transformers for the SIMPLE SWITCHER power converter family: AIE,

Pulse Engineering and Renco.  The capacitor database is built up using

the Sprague, Nichicon, Cornell-Dubilier and Panasonic lines.

2.4  Editing Component Values

You can override (i.e. edit) most component values of the list.

Some values cannot be edited: e.g. inductors, transformer turns

ratio and primary inductance, snubber components Rs and Cs,

resistor tolerances, etc.  Some values are adjusted automatically

with another component; e.g. R1 if R2 is changed, etc.

If you wish to use magnetics with characteristics other than

those automatically calculated by the program, you must indicate,

before the component limit values are calculated,  that you do

not want to use a "standard" inductor (or "automatically

calculated" transformer for the flyback regulator).  This is

covered further in Section 4.5.

The output capacitor ESR can be edited as an "attribute," by

hitting the space bar after the appropriate capacitor has been

highlighted (refer to Section 4.4.3).

2.5  Frequency Response Calculation

After the component selection is completed, the program

calculates the control loop's crossover frequency and phase

margin.  The design procedure ensures that the resulting circuit

is stable under worst-case conditions and has adequate phase

margin. The phase margin of the actual circuit will almost

invariably be better.  Should you require higher phase margin,

you can change component values (increase Co in LM2574/75/76-

based designs and increase  Co and Cc in LM2577-based designs)

and rerun the stability analysis to verify the improvement.  You

can also experiment with stability via component value  trade-

offs ( e.g. decreasing the value of the output capacitors and

checking the phase margin change). In this way you can

optimize the design to your own criteria. This is how we

optimized  the component values of the circuits you find in the

datasheet.

2.6  Thermal Calculations

Finally, a junction temperature check is performed, based on our

proprietary thermal model.  It calculates the junction

temperature of the chip in your application circuit with your

chosen package at the specified maximum ambient temperature.  If

the calculated junction temperature exceeds the maximum

temperature rating of the package (150 degC in K package and 125

degC in all others), the program specifies the maximum allowable

thermal resistance of the heatsink that must be used.  For the N

and M packages, where no heatsink is applicable, the minimum

required PC board copper area, attached to the ground pins, is

given.  Conversely, you can specify your heatsink/copper area and

the program will output the operating junction temperature at

Tamax.

2.7  Saving, Viewing, and Printing the File

After completing the design, provision is made for saving the

file, printing the schematic, and/or printing the components list

complete with manufacturers' part numbers. To print the

schematic, you must have an Epson dot matrix or HP LaserJet

compatible printer.  You may view the schematic with either

Hercules, CGA, EGA, or VGA graphics capabilities.

When saving the file, please specify a filename with a maximum of

six characters and no extension. The file will automatically be

saved on your current drive.  It is advisable to copy the program

to your hard disk before running it because the  floppy disk that

carries the program is quite full and does not have room for

stored circuits.  An added benefit of running from a hard disk is

that the program runs faster.

In the saving process, two files are created. One contains the

input specifications and limit values that you use when you

recall the design for more experimentation or to make some

changes. The other, stored under the name <filename>.LST contains

the final circuit specifications and the component list. This

file can be viewed from the program in case you do not have a

printer available (e.g. in a laptop environment). It is an ASCII

file, so you can edit it (add headers etc.) with any word

processor or text editor.

3.0  WARNING MESSAGES

The program displays warnings to alert you to certain conditions.

3.1  "Current or voltage limit exceeded."

This message appears after the feasibility check if either the

maximum current or voltage rating of the power switch in the IC

is exceeded.   You are required to take action should you receive

this message.  The specifications of the circuit have to be

modified (e.g. load or Vin max decreased) or, in the case of a

flyback converter, the turns ratio of the transformer may have to

be modified to stay within the IC's ratings.

3.2  "Burst mode operation."

This flag calls attention to the fact that due to a light load, the

regulator skips switching cycles to maintain regulation.  Strictly

speaking, the stability analysis is not valid in this operating

mode.  Practice shows, however, that the SIMPLE SWITCHER regulator

is stable under these conditions. The only possible problem is the

indeterminate emitted spectrum of the circuit, due to the non-

constant operating frequency. The burst mode warning will always

appear if a minimum load current is not specified, because the

default load value is zero.

3.3  "Short Circuit Runaway."

This condition can occur in flyback regulators.  The problem is

common to flybacks and not unique to the LM2577.  It indicates

that due to the nonzero minimum on-time of the power switch, the

switch current limit cannot be maintained in an output short circuit

condition.  This is because, during the minimum on-time, energy can

build up in the transformer that cannot be completely dissipated by

losses in the output diode/capacitor network.  This will eventually

cause the transformer core to saturate.  When the switch then turns

on at the next cycle, the current limit is not engaged fast enough

to protect the switch from the high current it must conduct from

saturated transformer.

Using a fast-recovery diode instead of a Schottky sometimes solves

the problem, because of its slightly greater power loss.  In

general, either Vin max or N (the transformer's turns ratio) has to

be decreased to avoid the problem and insure that the switch current

is safely limited in case of an output short circuit.

3.4  "Possible Subharmonic Oscillation."

In current-mode controlled DC/DC converters, like all the LM2577-

based converters, subharmonic oscillation can occur if the

compensating ramp slope is not high enough.  The built-in, fixed

compensating ramp of the LM2577 is designed so that it ensures

subharmonic oscillation-free operation in most practical

applications. However, in some cases the stability criteria may

be violated.  In these rare cases the program issues a warning.

Although the regulator maintains control of the output voltage

even under these circumstances, you may want to avoid this

operating mode, because of increased noise and output ripple.

The value of the inductor or the primary inductance of the

transformer has to be increased to avoid this condition.

3.5  "Maximum Duty-cycle Exceeded."

The duty-cycle of the LM2577 is limited to 90%, while for the

LM2574/75/76 it is limited to 93%.  If the specification requires

a higher duty-cycle, the program issues a warning.  Decrease the

output to input voltage differential or increase the

transformer's turns ratio to avoid this error condition in the

circuits using the LM2577.  In applications using the

LM2574/75/76, the minimum input voltage has to be increased.

4.0  GENERAL  COMMENTS

4.1  Standard Units

Throughout the program the following standard units are used: V,

A, OHM, H, F.  Any input quantity entered without unit

designation is understood in these units. E.g. a current "300" is

understood as 300A, if you want to input 300mA, you should type

"300m" or "300mA" or ".3".  "Micro" is understood by using the

suffix, "u". "Mega" is understood by using the suffix "M".

4.2  IC Options

In a buck or buckboost converter applications, if the specified

output voltage is 3.3, 5, 12 or 15V or -3.3, -5, -12 or -15V, the

program automatically chooses and displays the appropriate fixed

output voltage versions of the SIMPLE SWITCHER power converter family.

It also decides automatically whether the use of the LM2574, LM2575,

or the LM2576 family is indicated in the given application circuit,

based on the calculated peak switch current.

Similarly, for boost and flyback regulators with 12V or 15V

output voltage, the appropriate voltage option is automatically

selected.

For other output voltages the adjustable versions are specified

with appropriate voltage divider resistors of 1% tolerance.

If the specified ambient temperature limits are within the

temperature range -40 degC to 125 degC, the program automatically

specifies the LM2574/75/76 or LM2577 families.  For wider

temperature ranges, the LM1575/75 and LM1577 families in 4-lead

TO-3 package are specified. It should be noted, however, that the

IC itself is guaranteed to meet specifications in the -40 to 125

degC (LM2574/75/76 & LM2577) and the -55 to 150 degC (LM1575/76 &

LM1577) JUNCTION temperature range, with an approximate shutdown

junction temperature of 160 degC.

The user has the option to specify the package before the thermal

calculation.  If the design is not feasible in the chosen

package, the program recommends a different package (e.g the T

package instead of the M or N), and the thermal check is rerun.

4.3  Output Accuracy

The accuracy of the output voltage of both the buck and boost

converters using the fixed output voltage versions of the

LM2574/75/76 and LM2577 families is guaranteed to be better than

+/-5% over the entire line and load range, worst case component

variations and over the full temperature range.  If the

adjustable versions are used, the accuracy can be somewhat worse

depending on the tolerance of the external divider resistors

used.  If "perfect" divider resistors are used, the adjustables

actually have an accuracy specification slightly superior to that

of the fixed versions.

In buckboost and flyback converters the accuracy of the main

(directly regulated) output can be expected to meet the same

specification.  The auxiliary outputs of flyback converters are

regulated only by transformer coupling and usually have lower

output voltage accuracy due to their load regulation being much

worse than that of the prime regulated output.

4.4  Output Capacitor ESR

The ESR, equivalent series resistance, of the output filter

capacitors is a very important parameter in switching regulator

applications.  It introduces a zero in the regulator's control

loop.  To maintain good phase margin, the ESR has to be smaller

than a limit value in boost and flyback converters and has to be

between a high and a low limit in buck and buckboost regulators.

The program outputs these limits in the "Limit Values" section of

the screen.  They can be met using so-called "high frequency" or

"low ESR" capacitors, like the ones used in the internal

database.	If you choose your own capacitor from a different

manufacturer, make sure you use a product line that specifies the

ESR at a higher frequency than 120Hz, e.g. 10KHz or 100KHz.

Capacitors without high frequency ESR specifications are not

intended for use in switching regulators.

4.4.1  Effect of Temperature on Capacitor ESR

Aluminum electrolytic capacitor ESR is temperature dependent, and

it can increase substantially at low temperature.  If you design

a circuit that has to operate below 0 degC, you should consult a

detailed capacitor datasheet.

For operation below 0 degC, it is good practice to use a solid

tantalum capacitor (about 10% of the output capacitor's value) in

parallel with the aluminum electrolytic output capacitor to

minimize the effect of the increased ESR.

4.4.2  ESR and Output Ripple Voltage

The ESR has also a strong effect on the output ripple voltage of

the converter.  The ripple voltage can be calculated as the

product of the current ripple (of the inductor or transformer

winding feeding the capacitor) and the ESR.  In boost and flyback

converter applications the output ripple (Vripple) can be

specified as an input parameter.  The program ensures that the

specification is met by the final circuit.  The program both

alerts and disallows user choice of Vripple less than 0.005*Vout.

This is because very low values of ripple can only be achieved in

basic converter designs using impractically large output

capacitors with extremely low ESR. The practical solution to

obtaining very low ripple specs is to add a small L-C filter to

the output of the regulator (see the LM2576 datasheet), or to use

a linear post-regulator.  If Vripple is not specified, the

program assumes a ripple voltage of 0.01*Vout by default.

In buck and buckboost applications, the ripple voltage cannot be

explicitly specified.  It will be calculated by the program and

listed under "Limit Values".  The ripple voltage can be

decreased, if needed, by optionally selecting larger inductors or

lower ESR capacitors (within the limits) in these applications.

4.4.3  Editing Output Capacitor ESR

The program provides the capability to input your own capacitor

and ESR value (to enter ESR hit the space bar after you have

highlighted the capacitor in question in the component list).

After changing Cout and/or ESR, the program recalculates the

phase margin. It also recalculates the ripple on the output

automatically, and changes the value in the limit column. This

way, you can see immediately if by changing the ESR value the

maximum specified ripple voltage is exceeded.

4.5  Standard/Custom Inductors and Transformers

In each regulator you have the choice to use standard

inductors/transformers or to design your own (i.e. custom).

The "standard" inductors are chosen for 30% max current ripple

(at max load current, Ilmax).  For buck regulators with max load

current of 0.5A or less, the ripple current proportion is larger,

up to 90% for 100 mA max load regulators.  This helps minimize

the inductor size for very low power regulators, while

maintaining continuous operation (See Sections 4.8 and 4.9).

The standard transformers for flyback converters cover the input

voltage range 4.5V to 15V and the output voltage range of +/-10V

to +/-15V.

In the case of nonstandard flyback transformers you have two

choices: Either you can let the program "automatically calculate"

the characteristics of an appropriate transformer, or you can

provide the characteristics for a custom transformer, including

the value of the primary inductance (Lp) and turns ratio of the

main output (N1=Np/Ns1). In the former case, the transformer is

optimized to National's own set of criteria, built into the

program.

In either case you have to pay attention to the primary side

leakage inductance.  The standard transformers have 2% leakage

inductance, and this is the default value used for custom

transformers as well.  If you use full custom transformers, you

have the option to specify the leakage inductance as you input

the parameters of your transformer.  The snubber capacitor and

resistor are calculated based on this value.  If you

underestimate leakage, the stored energy in the primary leakage

inductance could destroy the power switch in the chip, especially

if your input voltage is high (higher than 15 - 20V). Should you

design your own transformer, it is good practice to wind it and

measure the leakage (by measuring the primary inductance with

shorted secondary).  You can use this measured value to input

into the program.  If you do not design/wind your transformer

carefully, you can expect leakage inductance on the order of 3%

to 5% of the primary inductance.  To decrease leakage, and

thereby decrease the dissipation in the snubber components, the

primary and secondary have to be interlayered.

4.6  Prime Output and Turns Ratio in Multiple Output Flyback

Converters.

The output designated by the "1" subscript (Vout1) is always the

"prime" or directly regulated output of the flyback converter.

The turns ratio of the secondary (output) windings are designated

N1, N2, and N3.  N1 = the number of secondary turns of output #1

divided by the number of primary turns; N2 and N3 are defined

similarly.  N2 and N3 are not variables you can change, they are

always chosen by the program, based on N1 and the output voltages

given in the input specifications section.

4.7  Crossover Frequency and Phase Margin

The program calculates the unity-gain crossover frequency and

phase margin of the converter's control loop. These parameters

characterize the loop's stability.  The calculation is done using

the final (edited) component values.  This feature enables the

user to fine tune the design and/or experiment with different

component values.

The phase margins of converters using the LM2577 are typically

high (60 - 80 deg), due to the chip's current-mode control

design. The phase margin calculations are performed under worst

case conditions, i.e. minimum temperature (-55 degC), and maximum

error amplifier transconductance.  As a consequence, the actual

crossover frequency is typically twice as high and the actual

phase margin also higher when typical parts are measured at room

temperature.

The buck and buckboost converters using the LM2574/75/76 have

typically lower phase margins. It should be noted however, that

their loop gain is extremely stable, and is practically constant

over process variations and operating temperature. This means

that the usual design reserves that account for loop gain

variations need not be applied, and a phase margin of 20 to 40

deg is perfectly acceptable.  Remember that the value the program

calculates is worst case.  The phase margin can be increased, if

needed, by increasing the value of the output capacitor, or in

the case of the buck converter, also by increasing the value of

the inductor. This can be done when editing in the "Component

Values" column. The program calculates the crossover frequency

and phase margin using the edited component values.  Editing the

ESR of the output capacitor can also have a positive effect on

the phase margin, but may increases the output ripple. It is

extremely useful that the program enables the user to edit ESR

and evaluate the design's sensitivity to this parameter.

4.8  Continuous/Discontinuous Operation

Every DC/DC converter can operate in either continuous or

discontinuous operating mode.  In continuous mode, the inductor

current (or the Magneto-Motive Force in transformers) never falls

to zero.  The converters operating in continuous mode are able to

deliver higher output power with the same power switch limits,

however, they also generate more radiated noise, need more output

filtering, and more careful layout.

For flyback converters, you may elect to implement your

specifications with a discontinuous mode converter. The program

warns you if this is not feasible and switches to continuous mode

automatically.

The program designs the boost converters ensuring continuous mode

operation, so as to maximize the available output power. You can

force discontinuous operation by choosing a low value custom

inductor.

For buck or buckboost converters, the program checks the input

parameters to decide whether the converter operates in continuous

or discontinuous mode. The model used to calculate crossover

frequency and phase margin is automatically adjusted to fit the

operating mode. The operating mode is indicated in the limit

column.

4.9  Low Power Converters and Discontinuous Operation

If very low output power is specified (Po<1W), discontinuous mode

operation yields superior results.  In these cases it is

recommended that you use custom inductors or full custom

transformers with adequately low primary inductance.  Should you

not use this preferred mode, the resulting continuous mode design

(although functional and stable) will have inductors and

capacitors with impractically high values.

Decreasing the inductor value in buckboost converters may

increase the value of the input inrush current at startup to

unacceptably high levels.  To avoid the problems associated with

this, you may use larger input capacitors (100 - 500uF) and a

delayed start-up via the ON/OFF pin.  The delay is accomplished

by a capacitor connected between this pin and the input supply

voltage, and a resistor connected between the pin and ground.

The undervoltage lockout circuit shown in the LM2576 datasheet

can also be used for this purpose.

It is worth noting that discontinuous operation usually yields

higher output ripple than its continuous counterparts, all other

parameters being equal.

4.10 Components Not Found in the Database

Components not found in the database are printed without vendor

callout in the component list. In the case of output capacitors,

the limit values are printed. In this instance, the worst case

ESR (from the limit column) is used for ripple and phase margin

calculations.

5.0  A WORD OF CAUTION:

Although we made every effort to ensure that the program is bug-

free and yields circuits that correspond to the specifications,

we strongly urge you to build the regulators and test them

thoroughly before using them in production.  National offers a

small PCB for evaluation purposes to assist you in evaluating

actual hardware.

6.0  QUESTIONS OR SUGGESTIONS

Our Design and Application staff have put a great deal of effort

into creating both a true Expert System and a user friendly tool. We

sincerely hope that you will find this diskette useful in applying our

SIMPLE SWITCHER power converter family of high performance, easy-to-

use voltage regulators.

"Switchers Made Simple" represents a breakthrough in design-aid

tools offered by semiconductor vendors.  Unlike modeling tools

that ANALYZE circuits the user must first create, this program

undertakes the more difficult task of SYNTHESIZING the design and

then calculating performance parameters.  It then takes the

additional step of identifying the actual components needed to

physically realize the circuit.  "Switchers Made Simple" is but

one part of a complete package of support that National offers

you, our customers.

Should you have any questions or suggestions regarding this

program, please call our Linear Applications (U.S.A) Group at

408-721-5615. You can also reach us by Fax at 408-721-7321.  For

international customers: in Europe, call 44-793-61-4141

(Swindon,England), in Japan, call 81-3-299-7001 (Tokyo), and in

S.E.A., call 852-733-1900 (Hong Kong).  In addition, please call

your local National Semiconductor sales office or any authorized

distributor for samples and evaluation boards.

7.0  SUGGESTED READING

1.   LM2574, LM2575, LM2576 and LM2577 datasheets.

2.   T. Szepesi, J. Bittner, H. Suzuki: Simple Switchers: A new

breed of power + control IC's for DC/DC converters. PCIM '89

Proceedings, 1989 Long Beach, pp:437-449

3.   T. Szepesi, J. Bittner, H. Santo: On card DC/DC converters

with the new "Simple Switcher" regulator family. PCIM '90

Europe Proceedings, 1990 Munich, pp 279-299.

4.   T. Szepesi, J. Bittner, H. Santo: Distributed power gets a

boost from the new Simple Switcher(TM) DC/DC coverter

family. PCIM Magazine, September 1990, pp.60-66.

To get a copy, call Linear Applications at the phone number above.

#