RF2HERC  Version 1.1

UPDATE

EQUIPMENT:

When using  "RF2HERC.COM" your  computer must  be  fitted

with a  Hercules Graphics  Adaptor  card  (or  compatible

card), which  is attached to a high resolution monochrome

monitor. The   monitor must be capable of a resolution of

720 X 348 lines.

RF2HERC uses 348 screen lines to display the fax

pictures, as opposed to 200 lines for the CGA version,

350 for the EGA version, and 480 lines for the VGA

version.

SOFTWARE DIFFERENCES TO OTHER VERSIONS OF RADFAX2:

When you  are in a RECEIVE WINDOW a certain number of fax

lines will  be superimposed  on each screen line in order

to squeeze  the total  number of  fax lines  that you are

saving onto  the 348  line screen.  When you are saving a

total of  1392  lines  or  less,  4  fax  lines  will  be

superimposed on  each screen  line. When  you are  saving

more than  1392 lines  in each  buffer, then  5 fax lines

will be  superimposed on  each screen line. This tends to

vertically squeeze  the picture  in  the  RECEIVE  WINDOW

(also in the RE-ALIGN WINDOW) when you have a buffer size

greater than 1392 lines.

TIMING OPTIONS:

RF2HERC gives  you the option of using either software or

hardware timing  for the reception of fax or  morse. RTTY

reception will  only  work  if  you  are  using  hardware

timing. This  option has  been added  as  some  computers

object to  the hardware  timing (some  models of  AMSTRAD

computers) method  used by  earlier  versions.  Only  use

software timing  if the hardware option doesn't work i.e.

your computer  locks up  when you  try to   receive a fax

picture.

To select  which timing  option to  use first  get into a

SETTINGS MENU  then toggle  the <T> key till the mode you

want is  displayed. Use  the <U>  UPDATE key to save your

selection  to   your  RADFAX2.SET  file  so  it  will  be

automatically loaded on next startup.

If using  software timing you will most certainly have to

reset the  DELAY and FINETUNE parameters of each speed so

that they  receive fax  at the correct rate. The best way

to do  this is to use a stop watch or clock with a second

hand  to  time  out  600  lines  of  fax  (minium  number

allowed). First  set the  #  LINES  =  parameter  in  the

SETTINGS MENU  to 600  by depressing  the <-> key. If you

want to  set speed  3 or  120 lines  per minute, you know

that each  line should take .5 of a second to scan in (60

seconds/120 lines)  therefore 600  lines should  take 300

seconds. Now go into a RECEIVE WINDOW by pressing the <B>

key from  the MAIN  MENU and  time how  long it  takes to

return to  the MAIN  MENU. If  it takes  longer than  300

seconds then  decrease the  DELAY value and try again. If

it takes  a shorter  time than  300 seconds then increase

the DELAY  value and try again. Keep doing this until you

come as  close as possible to a time of 300 seconds. When

you get  as close  as possible  to the  correct time goto

STEP 2 on page 10 of the RADFAX2 manual.

600 lines at 60 LPM should take 600 seconds to timeout.

600 lines at 90 LPM should take 450 seconds to timeout.

Don't forget  to update  your RADFAX2.SET  file with your

new TIMING, DELAY and FINETUNE parameters. Also note that

if  you  now  change  the  speed  (frequency)  that  your

computer is running (say from 4.77 Mhz to 10 Mhz, or from

10 Mhz  to 4.77  Mhz) then  these new  DELAY and FINETUNE

parameters will become  invalid ( so switch back).

CHANGE IN MORSE DECODER:

The morse decoder now uses the DELAY value of the 120 LPM

fax decoder as a timing reference. You don't have set the

fax speed  to SPEED  3 for  it to operate, as the program

knows where  to find  that variable  no matter  what  fax

speed you have set. Once you have set the DELAY value for

SPEED 3  (120 LPM)  correctly, and the TIMING method, the

program can  then use  that as a reasonably stable timing

base no matter whether you are running a 4.77 Mhz XT or a

16 Mhz  AT, using either SOFTWARE or HARDWARE TIMING. The

morse decoder  has been  optimised for  the 120 LPM DELAY

value, but it will still operate if this value differs by

a factor  of 4  ( DELAY*4,  DELAY/4)  as it has automatic

speed tracking.  You will  only  get  good  results  with

machine generated morse code. Set your receiver to the CW

mode or  the LSB  (lower sideband)  position. I  like the

audio output  set at  the lowest  level that the FSK/TONE

decoder can  tolerate. Too  high a volume will just swamp

the FSK/TONE decoder.

UPDATE INFORMATION FOR RADFAX2, RF2EGA, RF2VGA & RF2HERC

VERSION 1.6

1 st. Dec. 1990

FILES MENU:

CHANGING DISK DRIVES

To change disk drives, first you get into the FILES MENU from the MAIN

MENU, and then you press the <C> key. You will then be prompted to

enter a new DRIVE LETTER, which will then become the new default drive

(if it is a valid drive letter).

CHANGING DIRECTORIES

When you are in the FILES MENU you can change to another directory by

pressing the <N> key. You will then be prompted to enter (NOMINATE) a

new directory. Any valid DOS directory name will be accepted (.. , \ ,

etc.).

SETTINGS MENU

PRINTER SELECTION

When in the SETTINGS MENU you can select between Epson/IBM 9 pin, or

Epson 24 pin, or IBM 24 pin printers, by pressing the <P> key until

the selection you want appears on the screen. Use the <U>pdate key to

save this information in the RADFAX2.SET file.

OTHER INPUT PORTS

If you want to use an input port that the program doesn't support,

then you can do this by changing the first two bytes of the

RADFAX2.SET file. You will have to use a utility program such as

NORTON to modify the RADFAX2.SET file. The first byte in the file is

the least significant byte of the input port address, followed by the

most significant byte. This version of RADFAX2 will now display any

input port when the SETTINGS MENU, not just the ones that the program

supports, which was the case for earlier versions.

#