RADFAX2 Version 1.4 and RF2EGA Version 1.2 Update

The above  versions of  RADFAX2 now  have the  option  of

using  either   software  or   hardware  timing  for  the

reception of fax or  morse. RTTY reception will only work

if you  are using  hardware timing.  This option has been

added as  some computers  object to  the hardware  timing

(some models of AMSTRAD computers) method used by earlier

versions. Only use software timing if the hardware option

doesn't work  i.e. your computer locks up when you try to

receive a fax picture.

To select  which timing  option to  use first  get into a

SETTINGS MENU  then toggle  the <T> key till the mode you

want is  displayed. Use  the <U>  UPDATE key to save your

selection  to   your  RADFAX2.SET  file  so  it  will  be

automatically loaded on next startup.

If using  software timing you will most certainly have to

reset the  DELAY and FINETUNE parameters of each speed so

that they  receive fax  at the correct rate. The best way

to do  this is to use a stop watch or clock with a second

hand  to  time  out  600  lines  of  fax  (minium  number

allowed). First  set the  #  LINES  =  parameter  in  the

SETTINGS MENU  to 600  by depressing  the <-> key. If you

want to  set speed  3 or  120 lines  per minute, you know

that each  line should take .5 of a second to scan in (60

seconds/120 lines)  therefore 600  lines should  take 300

seconds. Now go into a RECEIVE WINDOW by pressing the <B>

key from  the MAIN  MENU and  time how  long it  takes to

return to  the MAIN  MENU. If  it takes  longer than  300

seconds then  decrease the  DELAY value and try again. If

it takes  a shorter  time than  300 seconds then increase

the DELAY  value and try again. Keep doing this until you

come as  close as possible to a time of 300 seconds. When

you get  as close  as possible  to the  correct time goto

STEP 2 on page 10 of the RADFAX2 manual.

600 lines at 60 LPM should take 600 seconds to timeout.

600 lines at 90 LPM should take 450 seconds to timeout.

Don't forget  to update  your RADFAX2.SET  file with your

new TIMING, DELAY and FINETUNE parameters. Also note that

if  you  now  change  the  speed  (frequency)  that  your

computer is running (say from 4.77 Mhz to 10 Mhz, or from

10 Mhz  to 4.77  Mhz) then  these new  DELAY and FINETUNE

parameters will  become  invalid ( so switch back). Morse

reception is unaffected by the TIMING option.